| Modifications for mcHF, compiled by DD4WH |  |  |  |
| --- | --- | --- | --- |
| use with care and at your own full risk! ;-) |  |  |  |
| 2015_09_14 |  |  |  |
|  | name of mod | what to modify | Reference |
| 1 | LCD HY28B to SPI | use headers to plug in LCD, do not solder to PCB, change jumpers on back of LCD | DF8OE yahoo-Group 13.9.2015 |
| 2 | bass response caps | change C26UI & C31UI from 470nF to 10µF for better audio bass response? | DD4WH not tested yet |
| 3 | U6 LDO | use LDO = low dropout regulator for U6 (UI_5V to UI_3V_MCU) | Clint 3.9.2015 |
| 4 | U5 LDO | use LDO = low dropout regulator for U5 (Vcc_5V to Vcc_3V) | Clint 3.9.2015 |
| 5 | decoupling DC UI_5V for the LCD supply | install C74a = 100µF and R35a = 4R7 on back of LCD; alternatively use chain of 2x 4R7 and 470µF cap --> Clint 3.9.2015 | yahoo group & Clint 3.9.2015; I am not sure if this is already part of board version 0.4? |
| 6 | omit RF att | omit Q2 and R40 | Clint 3.9.2015 |
| 7 | 47µF chokes | RFC5, RFC6, RFC7 have to be 47µH rather than 4.7 | Clint 3.9.2015 |
| 8 | watchout for polarization C5 & C6 | self-explanatory, maybe not relevant for board version 0.4 | Clint 3.9.2015 |
| 9 | OPA caps | place bridges or zero Ohm resistors instead of C71 & C73 | Clint 3.9.2015 |
| 10 | oscillation final amp | place a 10µF 16V tantalum cap in parallel with C106 (any value from 4.7µ to 22µ is fine) | Clint 3.9.2015 |
| 11 | PIN diode D3 | replace R53 with 220R and R54 with 3K9 | Clint 3.9.2015 |
| 12 | SWR measurement | bridge R59 & R60 with zero ohms; change R58 & R62 to 2K2; change C82 & C83 to 2µ2F; change R55 & R56 to 47µH INDUCTORS (4.7µH will work); capacitor (1n to 100nF) between pin 9 and pin 10 and ground (pin6) | Clint 3.9.2015 |
| 13 | T5 | BN 43-2402 small core | Clint 3.9.2015 |
| 14 | T7 | 2 x BN 43-202 large cores | Clint 3.9.2015 |
| 15 | T2 & T3 | wind with faraday-shielded Teflon coax | Clint 3.9.2015 |
| 16 | current through final transistors | change R74 & R76 to 2K2; install R73 & R75 standing!; 100nF caps to ground and to top of R73 & R75; top of these two to junction of R15 and Q1a (Clint) OR to PTT line=pin 1 of U1 (DF8OE) | Clint 3.9.2015 / DF8OE I40 homepage |
| 17 | gate resistors | change gate resistors R81 & R82 to 100R | DF8OE I40 homepage |
| 18 | decoupling of 8V | LM386 to +12V, not 8V; not yet explained in detail | DF8OE I40 homepage |
| 19 | PA bias | replace C96 with 100µF electrolytic or 22µF tantalum; put a 1K or 2K2 resistor in parallel with C96 | Clint 3.9.2015 |
| 20 | metal shield | install well insulated (need not be grounded) metal shield between RF and UI board | Clint 3.9.2015 |
| 21 | U5b | a separate 5V LDO regulator feeding the UI board; this is not further explained with schematics | Clint 3.9.2015 |